Iridex Diode Laser (532 nm)

“Ophthalmic”

PREOPERATIVE

Post laser signs

Cover all windows and exits with opaque drape.

Distribute glasses/goggles.

Plug laser into electrical outlet.

Attach foot pedal.

Turn on with key.

Note:  Laser will take approx. 10 min to warm-up.

INTRAOPERATIVE

FOR  FIBER USE:

Laser in 'Standby'

Ensure safety filter is installed onto the microscope

1. Remove the binocular assembly from the microscope

2. Install safety filter

3. Mount the binocular assembly on the safety filter

Connect the fiber to the laser

Eyewear must be worn by all personnel except those looking through the microscope.

FOR INDIRECT OPTHALMOSCOPE - HEADPIECE

Laser in 'Standby'

Connect LIO fiber and light source

Adjust illumination intensity

Adjust HeNe aiming beam intensity

Eyewear must be worn by all personnel except the Physician looking through the Indirect.

Set power, pulse time and repeat mode per physician.

Press READY only when surgeon requests laser.

Press STANDBY to change power or mode.

Laser stays in STANDBY when not in use.

## POST OPERATIVE

Record settings and number of shots.

Turn laser off.

Clean laser and accessories.

Store laser and accessories.

Remove laser signs from door.

Return Laser Log Sheet to book.